CSE 350 PROJECT

SECTION – 10

GROUP – 7

TANVIR MOHAMMAD SIAM

ID : 22101506

MAISHA ZAMAN KHAN

ID : 22301009

ONIK DEB NATH PARTHA

ID : 22301572

NUHAN HABIB

ID : 22101624

SYED MEHBUB MONJUR

ID : 22301033

Automatic Water Level Indicator with Motor Control

Maisha Zaman Khan                                  Tanvir Mohammad Siam                        ONIK DEB NATH PARTHA

Computer Science and Engineering       Computer Science and Engineering    Computer Science and Engineering

Brac University                                          Brac University                                        Brac University

Dhaka, Bangladesh                                  Dhaka, Bangladesh                                Dhaka, Bangladesh

Maisha.zaman.khan@g.bracu.ac.bd   Tanvir.mohammad.siam@g.bracu.ac.bd      onik.deb.nath.partha@g.bracu.ac.bd

Syed Mehbub Monjur                                      Nuhan Habib

Computer Science and Engineering               Computer Science and Engineering

Brac University                                                  Brac University

Dhaka, Bangladesh                                          Dhaka, Bangladesh

Syed.membub.monjur@g.bracu.ac.bd        nuhan.habib@g.bracu.ac.bd

Abstract—The Automatic Water Level Indicator with Motor Control is an embedded electronics project designed to address water management challenges such as overflow, dry pump running, and inefficient manual control. Utilizing fundamental concepts from digital electronics and pulse techniques, this system employs metal probes for water level sensing and NPN transistors with SR latch for automatic motor control. It provides visual water level indication through LEDs, activates a pump when water is low, and stops it when the tank is full, with a buzzer alert for overflow prevention. A diode ensures protection against inductive voltage spikes. This cost-effective solution showcases the practical application of digital electronics in real-world automation.

Index Terms—Water level indicator, motor control, transistor switching, SR latch, LED indication, embedded systems, digital electronics.

# I. INTRODUCTION

Water management is critical in residential and industrial settings to ensure resource conservation and device protection. Manual monitoring of tank levels is inefficient and errorprone, often resulting in overflow or dry running of water pumps. This project aims to automate the process using basic digital electronics, thereby minimizing human intervention while ensuring system reliability and operational safety.

# II. SYSTEM OVERVIEW

The system is built around three key water level probes—LOW, MID, and HIGH—installed inside a storage tank. These probes are constructed from conductive materials such as copper or steel. As water rises and touches each probe, the circuit is completed, lighting up the corresponding LED and indicating the current water level.

## A. Visual Indication

Each probe is connected to an LED via a 220Ω resistor to limit current and ensure safe operation. The LEDs offer realtime status updates, indicating whether the water level is low, mid, or high.

## B. Pump Control Logic

The pump control mechanism is implemented using an NPN transistor in a switching configuration. A 1kΩ resistor at the transistor’s base regulates the input current. When the LOW probe is uncovered, indicating an empty tank, the transistor receives a base signal and conducts, energizing the relay connected to a 12V DC motor.

# III. IMPLEMENTATION

The complete system is mounted on a breadboard using approximately twenty jumper wires for interconnection. Components include:

- 3 metal probes (LOW, MID, HIGH)

- 3 LEDs with 220Ω resistors

- 1 NPN transistor

- 1 diode

- 1 DC motor (pump)

- 1 buzzer

- SR latch

A 5V or 12V DC power supply is used based on component requirements. Various test conditions—empty tank, mid-filled, and full tank—were simulated to verify system performance.

# IV. RESULTS AND DISCUSSION

The system reliably detected water levels, triggered appropriate LED indicators, controlled the pump motor, and activated the buzzer at full capacity. The automatic motor control eliminated the need for manual monitoring, while the flyback diode ensured circuit protection. Overall, the system demonstrated robust logic and integration of digital electronics principles.

# V. CONCLUSION

The Automatic Water Level Indicator with Motor Control is a practical and economical solution for water tank automation. It eliminates overflow and protects the pump from dry running by automating the motor operation based on water level detection. Utilizing fundamental digital electronic components, the project successfully demonstrates how simple yet effective circuitry can solve real-world problems. The system also lays a foundation for future upgrades involving microcontrollers, wireless alerts, and IoT-based remote monitoring.